| Name: Click or tap here to enter text. | Date: Click or tap to enter a date. |
| --- | --- |

We saw previously that the inverting amplifier has a single input voltage, VIN, applied to the inverting input terminal. If we add more input resistors to the input we end up with another operational amplifier circuit called a summing amplifier, summing inverter or even a voltage adder circuit as shown below.

You showed in the prelab that the output voltage in the above circuit is related to the inputs according to

VOUT=-RfVIN1R1+VIN2R2+VIN3R3+⋯+VINnRn.

The summing amplifier is a very flexible circuit indeed, enabling us to effectively “add” or “sum”, hence its name, together several individual input signals. If the inputs resistors, R1, R2, R3 etc, are all equal a “unity gain inverting adder” will be made. However, if the input resistors are of different values a “scaling summing amplifier” is produced which will output a weighted sum of the input signals.

So what can we use summing amplifiers for? If the input resistances of a summing amplifier are connected to potentiometers the individual input signals can be mixed together by varying amounts.  For example, measuring temperature, you could add a negative offset voltage to make the output voltage or display read “0” at the freezing point.  Another common application would be an audio mixer for adding or mixing together individual waveforms (sounds) from different source channels (vocals, instruments, etc) before sending them combined to an audio amplifier as shown below.

# Adding a DC Offset to a Signal

The circuit below allows one to add a DC offset level to a signal as is commonly done in a function generator.  The original signal in experiences a gain of -1 and the offset signal has a gain of -1015=-23 so that the ∓15V range is converted to ±10V.

Build this circuit and try various input signals and see how that you can move the signal vertically up and down by adding the DC offset with the potentiometer.

Capture an oscilloscope trace where you have moved your input signal so that it is not centered on zero volts but not so much that the signal begins to clip.

# Digital to Analog Converter

A very basic digital-to-analog converter can be made with a weighted summing amplifier. Each successive bit in the digital word represents a level that is twice as large as the preceding bit. If each bit is taken as a given voltage, the increasing levels may be produced by using different gains in the summing inputs. A simple four-bit converter is shown to the right.

This system can create 24, or 16, different output voltage levels. Each of the four inputs are driven by a simple high/low logic level that represents a 1 or 0 for that particular bit. We will drive the four inputs with four digital output signals from the ELVIS II device.  These levels can be controlled by the ELVISmx Instrument named Digital Writer.  These outputs create 5V TTL-level signals where 5V represents a logical high (binary 1) and 0V represents logical low (binary 0).

Note that the input resistors vary by factors of 2. The gain for the lowest path is  R/R, or unity. This input is used for the most significant bit of the input word (MSB). The next input shows a gain of R/(2R), or 0.5. The third input shows a gain of 0.25, and the final (top) input shows a gain of 0.125. The final input has the lowest gain and is used for the least significant bit of the input word (LSB).

Build the above circuit, using resistors where R=1kΩ and connect the inputs to the first four digital output channels of the ELVIS II.  Use the Digital Writer to set the input value to each of the possible 16 values and measure and record the output levels that you obtain below.

| Input | Expected Output (V) | Measured Output (V) |
| --- | --- | --- |
| 0000 | Click or tap here to enter text. | Click or tap here to enter text. |
| 0001 | Click or tap here to enter text. | Click or tap here to enter text. |
| 0010 | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| 1111 | Click or tap here to enter text. | Click or tap here to enter text. |

# R/2R Ladder DAC

To increase resolution, it appears that all you need to do to the summing amplifier version of the digital to analog converter is add extra channels with larger and larger resistors. Unfortunately, the resistor sizes soon become impractical, and another approach is required. For example, a 16-bit system would require that the LSB resistor be equal to 65,536Rf.  One problem is that the resulting small input current may be dwarfed by input bias and offset currents. Also, high component accuracy is needed for the more significant inputs in terms of the input resistors and the drive signals. The excessively large resistors may also contribute added noise. The standard solution to this problem involves of the use of an R/2R  resistive divider network.

Analysis of the R/2R divider network is most easily done using Thevenin’s theorem.  A good discussion of how to do this is provided at the following microcontrollers lab web page.

https://microcontrollerslab.com/r-2r-ladder-dac-digital-to-analog-converter-working-examples-circuits/

Advantages of the R/2R ladder DAC over the binary-weighted summing approach include that it can be fabricated easily, it requires only two resistor values instead of a wide range of resistors, the output resistance remains the same despite the number of bits, and increasing the resolution does not degrade the performance.  However, one significant disadvantage is that it has a slow conversion speed.

Build a 3-bit version of the R/2R Ladder DAC and measure the output for each of the 23=8 inputs.

| Input | Expected Output (V) | Measured Output (V) |
| --- | --- | --- |
| 0000 | Click or tap here to enter text. | Click or tap here to enter text. |
| 0001 | Click or tap here to enter text. | Click or tap here to enter text. |
| 0010 | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| Click or tap here to enter text. | Click or tap here to enter text. | Click or tap here to enter text. |
| 111 | Click or tap here to enter text. | Click or tap here to enter text. |